AI FOR A
BETTER FUTURE

AN INTELLIGENT SMART CITY

Engineering Blueprint • System Architecture • Electronics • Source Code • Construction Plan

A proposed school science exhibition working model
Base concept: 90 cm × 60 cm

Build • Learn • Innovate

# Document status and scope

This document converts the proposed smart-city blueprint into an editable engineering plan. It is a design proposal, not a verified reproduction of the Instagram reel. The uploaded promotional PDF does not provide exact circuits, components, dimensions, or source code. Therefore, specifications below are proposed and must be checked against the exact parts purchased.

# Table of contents

1. Project overview

2. Master layout and dimensions

3. Module 1 — Healthcare

4. Module 2 — Environmental protection

5. Module 3 — Smart transportation

6. Module 4 — Safety and security

7. Central controller and wiring

8. Arduino firmware

9. Bill of materials and budget

10. Construction procedure

11. Testing and troubleshooting

12. Exhibition presentation and Q&A

13. Safety and final verification

# 1. Project overview

Purpose: demonstrate how sensors, automation, renewable-energy concepts, and optional AI software can support services in a miniature smart city. The working model combines sensor inputs with visible outputs such as traffic LEDs, automatic lighting, and a motion alert.

## Core modules

| Module | Physical elements | Demonstration |
| --- | --- | --- |
| Healthcare | Mini hospital, temperature sensor, status indicator | Show a sensor reading; this is not medical diagnosis. |
| Environment | Green area, solar-panel model, LDR streetlight | Streetlight responds to ambient light. |
| Transportation | Road junction, traffic LEDs, ultrasonic sensor | Traffic sequence and proximity detection. |
| Safety/security | Security building, PIR sensor, LED/buzzer | Movement triggers an alert. |
| Control center | Microcontroller and optional OLED | Centralized sensor/status demonstration. |
| Future technology | Information panel | Explain possible AI, data analysis, and connectivity upgrades. |

## AI versus automation

A microcontroller using fixed if/else rules performs automation. It is not automatically AI. A genuine AI upgrade requires a defined model, input data, inference code, and measurable output. The base model should work without an AI computer.

# 2. Master layout and dimensions

Proposed base: 90 cm long × 60 cm wide. Use a rigid board and leave access for wiring and repairs.

| Zone | Suggested position (x, y) | Approx. footprint |
| --- | --- | --- |
| Healthcare | x=5–35 cm, y=5–23 cm | 30 × 18 cm |
| Environment | x=55–85 cm, y=5–23 cm | 30 × 18 cm |
| Transportation | x=5–35 cm, y=34–52 cm | 30 × 18 cm |
| Safety/security | x=55–85 cm, y=34–52 cm | 30 × 18 cm |
| Control center | x=36–54 cm, y=45–58 cm | 18 × 13 cm |
| Info panel | front edge or rear backdrop | fit to remaining space |
| Main road | crosses between zones | 7–9 cm wide |

Coordinate origin: top-left corner of the base. Confirm the road and zone footprints physically before cutting materials.

## Layout sketch (not to scale)

REAR EDGE ┌────────────────────────────────────────────────────────┐ │ [01 HEALTHCARE] [02 ENVIRONMENT] │ │ Hospital / sensor Solar / park / LDR │ │ │ │══════════════ MAIN ROAD / INTERSECTION ═════════════════│ │ │ │ [03 TRANSPORT] [04 SAFETY & SECURITY] │ │ Traffic / parking PIR / alert │ │ [05 CONTROL CENTER] │ │ [06 INFORMATION PANEL] │ └────────────────────────────────────────────────────────┘ FRONT EDGE

# 3. Module 1 — Healthcare

Build a small hospital from foam board or cardboard; use removable roof panels if electronics are inside.

Use a compatible temperature sensor to display ambient or object-surface temperature, depending on the sensor.

Add an LED status indicator or optional OLED display.

Label the feature “temperature monitoring demonstration,” not “AI diagnosis.”

## Module wiring concept

| Part | Connection concept | Notes |
| --- | --- | --- |
| Temperature sensor | Power, ground, and signal to a compatible input | Exact pinout depends on the selected sensor. |
| Status LED | GPIO output through a current-limiting resistor | Choose a resistor for the LED and board voltage. |
| OLED (optional) | I²C SDA/SCL, power, ground | Confirm display voltage and I²C address. |

# 4. Module 2 — Environmental protection

Build a green park with miniature trees and a solar/wind-energy display.

Use an LDR voltage divider to detect ambient light and switch streetlight LEDs.

Treat the miniature solar panel as an energy demonstration unless the power budget proves it can run other loads.

If adding a turbine motor, use a suitable driver/transistor circuit and a separate supply where required.

## LDR circuit concept

5V ── LDR ──┬── Analog input (A0) ¦ 10 kΩ ¦ GND LED output pin ── suitable resistor ── LED ── GND Note: the divider order affects whether the analog value rises or falls in darkness.

# 5. Module 3 — Smart transportation

Mark lanes, zebra crossings, road dividers, and miniature parking bays.

Use red/yellow/green LEDs for a simple traffic sequence.

Use an ultrasonic sensor for proximity demonstration; it does not identify a vehicle type.

Use a push button to simulate an emergency-priority request.

Do not connect this model to real traffic equipment.

## Traffic logic

START ↓ Read inputs ↓ Emergency button pressed? ├─ Yes → run a defined, safe demo priority sequence └─ No → run normal traffic sequence ↓ Update parking / proximity status ↓ Repeat

# 6. Module 4 — Safety and security

Install a PIR sensor in a visible position near the model security building.

Trigger an LED or optional buzzer when motion is detected.

A PIR detects changes in infrared radiation; it does not identify a person.

Keep alerts low-voltage and suitable for a classroom demonstration.

# 7. Central controller and wiring

A single Arduino Uno-compatible board may run the basic demonstrations, but the final pin map must be checked against the exact sensor/display choices. The pin assignment below is a proposed example, not a verified wiring plan for unspecified modules.

| Function | Example pin | Important note |
| --- | --- | --- |
| LDR divider | A0 | Analog input. |
| Temperature sensor | A1 or a suitable digital pin | Depends on sensor type. |
| PIR output | D2 | Check the module's output voltage. |
| Ultrasonic TRIG | D3 | Digital output. |
| Ultrasonic ECHO | D4 | Check logic-level compatibility. |
| Traffic LEDs | D5, D6, D7 | Each LED requires a resistor. |
| Streetlight LED | D8 | Use suitable current limiting. |
| Security alert LED | D9 | Use suitable current limiting. |
| Emergency button | D10 | Can use INPUT_PULLUP. |
| OLED (optional) | I²C SDA/SCL | Uno commonly uses A4/A5; verify board. |

## Power rules

Use USB or an appropriate regulated low-voltage supply for the controller.

Do not power motors directly from GPIO pins.

Use a suitable driver and supply for motors or high-current loads.

Connect grounds appropriately when required by the circuit design.

Never connect household mains electricity to the model.

# 8. Arduino firmware

The following sketch implements a basic demonstrator: LDR streetlight, PIR alert LED, ultrasonic proximity reading, and a timed three-LED traffic sequence. It does not implement a medical display, OLED, motor drive, smart-parking indicator, or a safety-certified emergency traffic controller. Test each circuit before use.

## File: SmartCity.ino

// AI for a Better Future — basic sensor/automation demonstrator // Target: Arduino Uno-compatible board // Example pin map; verify against your actual modules before wiring. const byte LDR_PIN = A0; const byte PIR_PIN = 2; const byte TRIG_PIN = 3; const byte ECHO_PIN = 4; const byte RED_LED = 5; const byte YELLOW_LED = 6; const byte GREEN_LED = 7; const byte STREET_LED = 8; const byte ALERT_LED = 9; const int DARK_THRESHOLD = 450; // Calibrate for your LDR divider const unsigned long TRAFFIC_STEP_MS = 3000; const unsigned long PULSE_TIMEOUT_US = 25000; unsigned long lastTrafficChange = 0; byte trafficState = 0; void setTraffic(byte state) { digitalWrite(RED_LED, state == 0 ? HIGH : LOW); digitalWrite(GREEN_LED, state == 1 ? HIGH : LOW); digitalWrite(YELLOW_LED, state == 2 ? HIGH : LOW); } float readDistanceCm() { digitalWrite(TRIG_PIN, LOW); delayMicroseconds(2); digitalWrite(TRIG_PIN, HIGH); delayMicroseconds(10); digitalWrite(TRIG_PIN, LOW); unsigned long duration = pulseIn(ECHO_PIN, HIGH, PULSE_TIMEOUT_US); if (duration == 0) return -1.0; // No echo / out of range return duration * 0.0343f / 2.0f; } void setup() { pinMode(PIR_PIN, INPUT); pinMode(TRIG_PIN, OUTPUT); pinMode(ECHO_PIN, INPUT); pinMode(RED_LED, OUTPUT); pinMode(YELLOW_LED, OUTPUT); pinMode(GREEN_LED, OUTPUT); pinMode(STREET_LED, OUTPUT); pinMode(ALERT_LED, OUTPUT); Serial.begin(9600); setTraffic(0); lastTrafficChange = millis(); } void loop() { // LDR: if your divider is reversed, invert this comparison. int lightValue = analogRead(LDR_PIN); digitalWrite(STREET_LED, lightValue < DARK_THRESHOLD ? HIGH : LOW); // PIR motion alert bool motionDetected = digitalRead(PIR_PIN) == HIGH; digitalWrite(ALERT_LED, motionDetected ? HIGH : LOW); // Proximity measurement float distanceCm = readDistanceCm(); // Simple educational traffic sequence: red -> green -> yellow -> repeat. unsigned long now = millis(); if (now - lastTrafficChange >= TRAFFIC_STEP_MS) { trafficState = (trafficState + 1) % 3; setTraffic(trafficState); lastTrafficChange = now; } Serial.print("LDR="); Serial.print(lightValue); Serial.print(" Motion="); Serial.print(motionDetected ? "YES" : "NO"); Serial.print(" Distance_cm="); if (distanceCm < 0) Serial.println("NO_ECHO"); else Serial.println(distanceCm, 1); delay(100); // Short sampling pause; not suitable for safety-critical control. }

## Upload steps

Install Arduino IDE from the official Arduino website.

Connect the board with a USB data cable.

Select the correct board and serial port.

Wire the circuit with power disconnected; check polarity and pin assignments.

Compile the sketch, resolve any board/library errors, then upload.

Open Serial Monitor at 9600 baud and check sensor readings.

Calibrate DARK_THRESHOLD by observing readings in bright and dim conditions.

Test the traffic LEDs, streetlight, and PIR alert separately before integrated operation.

## Known limitations

The traffic sequence is a simple classroom demonstration and does not use vehicle detection to adapt timing.

The ultrasonic distance is reported to Serial Monitor only.

PIR sensors may need a warm-up period and can produce false triggers.

The LDR threshold depends on ambient light, resistor value, and divider orientation.

For a real emergency-priority simulation, design and test a state machine that prevents conflicting green signals.

# 9. Bill of materials and budget

| Item | Qty. | Estimated cost (₹) | Essential? |
| --- | --- | --- | --- |
| Rigid baseboard 90 × 60 cm | 1 | 250–500 | Yes |
| Foam board/cardboard | As needed | 150–300 | Yes |
| Artificial grass, trees, craft scenery | 1 set | 200–400 | Yes |
| Arduino Uno-compatible board | 1 | 400–800 | Yes |
| Breadboard and jumper wires | 1 set | 150–250 | Yes |
| LEDs and resistors | 1 set | 80–150 | Yes |
| LDR and fixed resistor | 1 set | 20–60 | Yes |
| PIR motion sensor | 1 | 50–120 | Yes for security module |
| Ultrasonic sensor | 1 | 80–150 | Optional for proximity |
| Temperature sensor | 1 | 50–150 | Optional for healthcare |
| Push button and connectors | 1 set | 50–100 | Optional |
| OLED display | 1 | 150–300 | Optional |
| Miniature vehicles/signs | 1 set | 200–400 | Recommended |
| Paint, glue, finishing materials | 1 set | 200–400 | Yes |

Budget estimate: roughly ₹1,880–₹4,080 for this listed set, depending on selection and local prices. This is not a live quote. Reuse materials where possible. Add delivery, spare wires, and tools if needed.

# 10. Construction procedure

Draw the 90 × 60 cm base outline and mark zones and road corridors.

Build the road surface and crossings before placing buildings.

Fabricate removable buildings and label each module.

Place the controller where it remains accessible for USB connection and repair.

Build and test each circuit on a breadboard before installing it in the model.

Route wires underneath the base through planned openings; avoid sharp edges and moving parts.

Install sensors so their fields of view are not blocked by scenery.

Upload the firmware and test each feature independently.

Secure components only after the electrical checks pass.

Perform the full demonstration several times and prepare a backup explanation if a sensor fails.

# 11. Testing and troubleshooting

| Problem | Likely cause | Action |
| --- | --- | --- |
| Board not detected | Charge-only USB cable, wrong port, missing driver | Use data cable; check port/board selection. |
| Streetlight always on/off | Threshold or LDR divider orientation | Read analog value in Serial Monitor and calibrate. |
| PIR false triggers | Warm-up, placement, power noise | Wait for warm-up; reposition and check supply. |
| Ultrasonic shows NO_ECHO | Wiring, target angle, range | Check TRIG/ECHO and test against a flat target. |
| LED does not light | Polarity, missing resistor, wrong pin | Check wiring and pin definition. |
| Board resets | Supply overload or short circuit | Disconnect loads and test one module at a time. |
| Code won't compile | Board selection or syntax issue | Read the first compiler error and verify the sketch. |

# 12. Exhibition presentation and Q&A

## Sample 2-minute presentation

Good morning. Our project is “AI for a Better Future: An Intelligent Smart City.” It demonstrates how technology can support healthcare, environmental sustainability, transportation, and public safety. The model uses sensors to collect information and a microcontroller to control outputs. For example, an LDR can switch a streetlight based on ambient light, a PIR sensor can trigger a motion alert, and ultrasonic sensing can demonstrate object proximity. These features are examples of sensing and automation. A genuine AI extension would require a defined AI model and a task such as classifying sample data or explaining sensor trends. Our model is educational and is not intended to control real infrastructure. It shows how carefully designed systems can make cities more responsive, efficient, and sustainable. Thank you.

## Likely judge questions

| Question | Answer |
| --- | --- |
| Is every part AI? | No. The basic build demonstrates sensing and automation. AI is an optional, separately implemented extension. |
| How does the LDR work? | Its resistance changes with light, changing the voltage read by an analog input. |
| What does the PIR detect? | Changes in infrared radiation associated with movement in its field of view. |
| Does the model diagnose illness? | No. It may demonstrate temperature measurement only. |
| How could AI be added? | Connect sensor data to a computer or Raspberry Pi and implement a defined, testable inference task. |
| Can it run real traffic lights? | No. It is a low-voltage classroom model, not a safety-certified traffic system. |

# 13. Safety and final verification

All power sources are low-voltage and suitable for the selected components.

No motor is powered directly from a GPIO pin.

LEDs have appropriate current-limiting resistors.

All wiring matches the final pin table and source code.

USB and power connections remain accessible.

No bare conductors can short against metal or each other.

Buildings are stable and do not obstruct sensor operation.

Every implemented feature has been tested individually.

All optional or conceptual features are labeled honestly.

Budget and component availability have been checked before purchase.

# Final design note

This is an editable first engineering document. Before buying components, confirm the exact Arduino board, sensor models, display type, power needs, and physical dimensions. The example firmware is a starter demonstrator and should be reviewed against the actual hardware; it is not a certified control system.